APJII Indonesian Internet eXchange Point

Technical Design and Configuration

Barry Raveendran Greene

bgreene@cisco.com

Tony S. Hariman

tony@cbn.net.id

INTRODUCTION

The Indonesian Internet eXchange (iIX) is a collaborative project sponsored by APJII. It's overall objective is the keep all intra-Indonesian Internet traffic within Indonesia. The iIX will obsolete the old method of using the US and other countries to exchange intra-Indonesian Internet traffic. iIX creates the foundation for a comprehensive national Internet infrastructure where all ISP collaborate for the common good.

TECHNICAL OVERVIEW

The iIX interconnects three different sites to make one large IXP.  The three sites are Telkom Indonesia, SatelIndo, and Indosat. ISPs can choose which site to connect based on circuit availability, upstream customer relationships, or organizational preference.

IPv4 Address Plan

IPv4 addresses have been assigned from the IANA IXP coordinator - Bill Manning. One /24 has been assigned for the entire IXP. Hence, CIDR compliance is required for any router or switch connecting to the IXP and all ISP's equipment working with the iIX.  198.32.204.0/24 has been assigned. The following is a break down of the assignments in the iIX (current on the data of this document):

| IP Address Block | Assignment |
| --- | --- |
| 198.32.204.0/28 | Loopback Interfaces on the routers |
| 198.32.204.1 | Telkom Loopback Interface |
| 198.32.204.2 | Satelindo Loopback Interface |
| 198.32.204.3 | Indosat Loopback Interface |
| 198.32.204.16/28 | Point to Point Serial Links ( 4 x /30s) |
| 198.32.204.16/30 | Telkom  Satelindo Link |
| 198.32.204.20/30 | Telkom  IndoSat Link |
| 198.32.204.24/30 | Satelindo  IndoSat Link |
| 198.32.204.32/27 | Point to Point Serial Links ( 8 x /30s) |
| 198.32.204.32/30 | Cbn.net.id Link |
| 198.32.204.36/30 | Mega.net.id Link |
| 198.32.204.64/26 | Telkom IX Medium (ethernet) |
| 198.32.204.65 | iIX-Tel Router Ethernet port |
| 198.32.204.66 | Indo.net.id's router ethernet port |
| 198.32.204.67 | Pacific.net.id's router ethernet port |
| 198.32.204.125 | Traffic Probe |
| 198.32.204.126 | Logging & Configuration Server |
| 198.32.204.128/26 | IndoSat IX Medium (ethernet) |
| 198.32.204.129 | iIX-IND Router Ethernet Port |
| 198.32.204.130 | Indosat's Customer Router Ethernet Port |
| 198.32.204.131 | Indosat's Core Router Ethernet Port |
| 198.32.204.189 | Traffic Probe |
| 198.32.204.190 | Logging & Configuration Server |
| 198.32.204.192/26 | Satelindo IX Medium (Ethernet) |
| 198.32.204.193 | iIX-SAT's router ethernet port |
| 198.32.204.194 | Satelindo's router ethernet port |
| 198.32.204.253 | Traffic Probe |
| 198.32.204.254 | Logging & Configuration Server |
| End | end |

Autonomous Systems Number

AS7597 has been delegated from APNIC for the iIX. This will be used across the IXP for inter-IXP traffic (between the three sites) and with IXP members (in route reflector mode) who have not yet received their own ASN from APNIC.  All IXP members are encouraged to get their own ASN.

HOW TO GET YOUR OWN ASN ? Organizations in the Asia & Pacific region get their Autonomous Systems Number (ASN) from APNIC (http://www.apnic.net). To apply to get your own ASN, you first must be either a member of APNIC or you will need to pay a one time charge and annual maintenance fee for the ASN. Current Internet policies regarding ASN allocations are documented in RFC 2050 (ftp://ds.internic.net/rfc/rfc2050.txt).  APNIC-048 template (ftp://ftp.apnic.net/apnic/docs) will need to be filled in and submitted. Fill in the as-in as-out as shown below:

as-in:	7597  ANY

as-out:	THIS-AS ANY

Explanation:

as-in:	7597  ANY     !  receive any routing information from AS 7597 ( IIX AS number)

as-out:  THIS-AS ANY    !  send any routing information from your as number.

After completing this form, please submit it via email to:

as-request@rs.apnic.net

After you get the AS number please contact  iIX admin for changing the bgp peering configuration from IBGP to EBGP.

Route Filtering via BGP

Filtering routes via BGP is necessary to enforce the IXP's peering policy and to protect the members of the IXP from unwanted routes. Two filtering techniques are used in this configuration: AS Path Filters and Distribute Lists Filters.

AS Path Filters

AS Path Filters will be used to enforce the policy for the IXP. Only the ASNs with belong to the members of the IXP will be distributed to the IXP members. This explicit permit rule will, by default, deny ASNs which are not explicitly permitted in the AS Path Filter. This prevents routes leaking from one provider from being propagated across the IXP. For example, if provider A was connected to Internet MCI (AS3561) in the US, and for some reason Internet MCI's routes leaked through provider A, then the AS Path Filters would block AS3561 since they are not explicitly in the AS Path list.

In addition, the AS Path filters will use a format that will only accept routes that originate from the ISP peering to the IXPs. For example, _7597$ will only allow routes which originate in AS7597. Any routes that originate outside of 7597 (for example Internet MCI) will be denied.

Lastly, the AS Path filters will be applied on all outbound BGP peers for both the IBGP route reflector connections (for those ISPs with out their own ASN) and the EBGP peer connections (for those ISP with their ASN). AS Path filters will also be applied on all EBGP inbound connections.

Distribute List Filters

Distribute List filters will be used to remove RFC 1918 (Private Address Space), multicast, and other routes that should not be propagated on the Internet routing table (i.e. like 127.0.0.0/16).  This safe guard keeps networks link 10.0.0.0/8 from getting leaked out across the Internet.

The following is the BGP ingress/egress route recommended filter:

access-list 180 deny   ip host 0.0.0.0 any

access-list 180 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 180 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 180 permit ip any any

IP Filtering

IP filtering is an additional policy enforcement tool. Essentially, you place a egress and ingress ip packet filter on the peer connections connecting to the IXP.  Every packet coming into or out of the IXP will get checked.  Some filters are designed to insure that all packets that leave a ISP belong to that ISP. This called ingress filtering. Other filters check packets coming into a ISP are not "rouge," private, or spoofed addresses. This is called egress filtering.  ISPs who connect to an IXP are encouraged to do both.

The following is an example of ISP connecting to an IXP. Egress and ingress ip packet filters are applied. The ingress filters insure that only packets with source address in the ISP's  CIDR block(s) are advertised out. The egress filters prevent spoofing, rouge addresses, and private addresses from coming into the ISP.

NOTE: There could be a performance impact on the forwarding speed of the router when a lot of filters are applied. Cisco's newer switching technologies minimize the performance impact of IP filtering. For example, Netflow switching is extremely efficient minimizing the performance impact of very long IP access list. Due care and consideration should be taken when ever the access list start getting beyond 50 entries. Though, it should be stated that trading a few microsecond of IP forwarding vs opening yourself to certain denial of service attacks may prove to be worth it the few microseconds.

Ingress Filtering  - Preventing Transmission of Invalid IP Addresses

By filtering packets on your routers that connect your network to the Internet, you can permit only packets with valid source IP addresses to leave your network and get into the Internet. For example, if your network consists of network 165.21.0.0, and your router connects to your ISP using a serial 0/1 interface, you can apply the access-list as follows:

access-list 110 permit ip 165.21.0.0 0.0.255.255 any

access-list 110 deny ip any any log

interface serial 0/1

ip access-group 110 out

Figure 1 - Ingress Filtering

The last line of the access-list determines if there is any traffic with an invalid source address entering the Internet. If there are any matches, they will be logged. It is not crucial to have this line, but it will help locate the source and extent of the possible attacks.

Egress Filtering - Preventing Reception of Invalid IP Addresses

For ISPs who provide service to end networks, we highly recommend the validation of incoming packets from your clients. This can be accomplished by the use of inbound packet filters on your border routers. For example, if your clients has a network number of 165.21.0.0/16, your should not seen any packets coming into your network with 165.21.0.0 in the source. These packets are attempts at spoofing and should be dropped.  The following example shows a sample filter for network 165.21.0.0 with filters for private and rouge routes:

access-list 111 deny ip host 0.0.0.0 any log

access-list 111 deny ip 127.0.0.0 0.255.255.255 any log

access-list 111 deny ip 10.0.0.0 0.255.255.255 any log

access-list 111 deny ip 172.16.0.0 0.15.255.255 any log

access-list 111 deny ip 192.168.0.0 0.0.255.255 any log

access-list 111 deny ip 165.21.0.0 0.0.255.255 any log

access-list 111 permit ip any any

interface serial 1/0

ip access-group 111 in

Figure 2 - Egress Filtering

All the "anti spoof," private address, and rouge filters have log any matches. If there are any, they would be logged. It is not crucial to have this line, but it will help locate the source and extent of the possible probes or attacks.

Putting it all together.

The following is a ISP allocated CIDR block 165.21.0.0/16 with both filters on the Interface:

interface Serial 0

description Connection to Upstream ISP

ip address XXX.XXX.XXX.XXX 255.255.255.252

no ip redirects

no ip directed-broadcast

no ip proxy-arp

ip route-cache same-interface

ip access-group 111 in

ip access-group 110 out

access-list 110 permit ip 165.21.0.0 0.0.255.255 any

access-list 110 deny ip any any log

access-list 111 deny ip host 0.0.0.0 any log

access-list 111 deny ip 127.0.0.0 0.255.255.255 any log

access-list 111 deny ip 10.0.0.0 0.255.255.255 any log

access-list 111 deny ip 172.16.0.0 0.15.255.255 any log

access-list 111 deny ip 192.168.0.0 0.0.255.255 any log

access-list 111 deny ip 165.21.0.0 0.0.255.255 any log

access-list 111 permit ip any any

Connecting to the IXP

iIX will have several types of ISPs connecting to the IXP fabric. Some ISPs will have their own Autonomous System Numbers (ASNs) and will peer directly to the IXP. Other ISPs will peer directly, but have yet obtained a ASN. These will connect via BGP route reflectors within the iIX's own ASN. Still others will connect through the commercial Layer 3 IXPs of Indosat and Satelindo. All of these ISPs will have to integrate BGP into their internal network routing topology. This section's goal is to highlight some of the design and integration options available to the ISP connecting to the iIX.

ISPs with no ASN

Some ISPs will not have their own ASN. The are connected to the Internet via a lease line to a Network Service Provider (NSP) in the US. Their internal routing could be built with static routes, OSPF, IS-IS, EIGRP, or RIPv2. What follows is one example of a way a ISP can configure their routing protocols.

Figure 3 - ISP with no ASN

ISP1 is a typical ISP in their early deployment. They get one connection to an upstream NSP (usually in the US), get allocated a block of addresses - 171.16.0.0/19 from their IPv4 registry, and  use OSPF in their internal backbone routing protocol. In this example, the internal backbone is a ethernet switch with 171.16.1.0/24 assigned for devices connecting to the backbone.. A static route pointing the default to their upstream provider is used to provide proper routing within their network. The NSP take their CIDR block and advertises it to the rest of the Internet. The configs on routers RTA and RTG might look something like the following:

RTA

interface Ethernet 0

description Internal Backbone link to RTG

router ospf 1

network 171.16.1.0 0.0.0.3 area 0

redistribute static subnets

RTG

interface Serial 0

description Link to Upstream NSP

interface ethernet 0

description Internal backbone link to RTA

router ospf 1

network 171.16.1.0 0.0.0.3 area 0

redistribute static subnets

ip route 0.0.0.0 0.0.0.0 Serial0

Config Example 1 - ISP with out an ASN using OSPF as IGP

This simple OSPF config demonstrates how RTG is inserting a static default into OSPF for the rest of ISP1's backbone.  If the link to the upstream provider is lost, then the static will drop and ISP1 will loose their default. Traffic will still be routed within their network.

How does this connect to a IXP link iIX? Especially since ISP1 does not have their ASN? Simple. You connect RTA to the IXP and include it in the ASN for iIX. While not optimal, this works until ISP1 gets their own ASN.

Figure 4 - ISP using Route Reflector to Connect to IXP

In this example, RTA's Serial0 interface is connected to the iIX via a lease line. RTA becomes a member of the iIX ASN, 7597. Routed received from AS7597 are redistributed into OSPF. All other routers in ISP1 would get a list of all the routes from iIX. Hence, all traffic bound for a member of iIX would go out that iIX  RTA link. All other routes would be forward to default.

Here is an example config:

RTA

interface serial 0

Description link to iIX

p address 172.17.1.1 255.255.255.252

interface Ethernet 0

description Internal Backbone link to RTG

ip address 172.16.1.2 255.255.255.0

router ospf 1

network 171.16.1.0 0.0.0.255 area 0

network 171.17.1.0 0.0.0.3 area 0

redistribute static subnets

redistribute bgp

router bgp 7597

no synchronization

bgp dampening

network 172.16.0.0 mask 255.255.192.0

neighbor 172.17.1.2 remote-as 7597

neighbor 172.17.1.2 next-hop-self

neighbor 172.17.1.2 version 4

neighbor 172.17.1.2 distribute-list 180 in

neighbor 172.17.1.2 distribute-list 180 out

neighbor 172.17.1.2 route-map IIX-IN in

no auto-summary

ip as-path access-list 1 permit _7597_

route-map IIX-IN permit 10

match as-path 1

ip route 172.16.0.0 255.255.192.0 Null0

RTG

interface Serial 0

description Link to Upstream NSP

interface ethernet 0

description Internal backbone link to RTA

ip address 172.16.1.1 255.255.255.0

router ospf 1

network 171.16.1.0 0.0.0.255 area 0

network 171.16.0.0 0.0.31.255 area 0

default-information originate

redistribute static subnets

ip route 0.0.0.0 0.0.0.0 Serial0

Config Example 2 - IBGP (ISP without an ASN)

Locking your BGP Network Advertisements UP

Some configuration techniques on the Internet have been developed over time to insure ISPs are being good Internet citizens and minimizing the effect their network has on the rest of the Internet. Locking BGP network advertisements up is one of these configuration techniques. Normally, BGP would be dynamic, sensing if a network is up or down and sending update accordingly. Hence, internal changes in your network would be propagate throughout the Internet. This is commonly referred to as route flapping and is not a good thing for the Internet.

The commonly used BGP config technique is to lock your BGP CIDR advertisements up is to include a status route for the CIRD block pointing to Null0. For example, if you have a BGP network advertisement like:

router bgp 7597

network 172.16.0.0 mask 255.255.192.0

then you would lock the route up with a static:

ip route 172.16.0.0 255.255.192.0 Null0

BGP to OSPF to BGP

ISPs who are already using BGP to connect to the Internet should not have any problem connecting to iIX. Due consideration must be paid to the filters used. ISPs should insure that iIX routes are not leaked to the rest of the Internet and routes from the Internet are not leaked into iIX. Figure 5 is a diagram of an ISP (AS 100) with connections to two upstream NSPs (AS 200 & AS 300). EBGP is used to advertise the ISP's route objects to iIX, NSP2 and NSP3. The full Internet routing table is pulled from the two NSPs.

Proactive filtering is a must in this situation. Config Example 3 is one example of BGP filtering that would satisfy the requirements the filtering requirements. Note that the as-path access-list 3 filters routes that originate in AS 100. This is to prevent routes advertised out one NSP from coming back through the other NSP.

Other filter rules will also work. Refer to the documentation on BGP Regular Expression filters.

RTA

interface serial 0

Description link to iIX

ip address 172.17.1.1 255.255.255.252

interface Ethernet 0

description Internal Backbone link to RTG

ip address 172.16.1.2 255.255.255.0

router ospf 1

network 171.16.1.0 0.0.0.255 area 0

network 171.17.1.0 0.0.0.3 area 0

redistribute static subnets

router bgp 100

no synchronization

bgp dampening

network 172.16.0.0 mask 255.255.192.0

neighbor 172.16.1.1 remote-as 100

neighbor 172.16.1.1 version 4

neighbor 172.17.1.2 remote-as 7597

neighbor 172.17.1.2 version 4

neighbor 172.17.1.2 distribute-list 180 in

neighbor 172.17.1.2 distribute-list 180 out

neighbor 172.17.1.2 route-map IIX-IN in

neighbor 172.17.1.2 route-map IIX-OUT out

no auto-summary

ip as-path access-list 1 permit _7597_

ip as-path access-list 2 permit _100$

ip route 172.16.0.0 255.255.192.0 Null0

route-map IIX-IN permit 10

match as-path 1

route-map IIX-OUT permit 10

match as-path 2

RTG

interface Serial 0

description Link to Upstream NSP #1

ip address 192.68.1.1 255.255.255.252

interface Serial 1

description Link to Upstream NSP #2

interface ethernet 0

description Internal backbone link to RTA

ip address 172.16.1.1 255.255.255.0

router ospf 1

network 171.16.1.0 0.0.0.255 area 0

network 171.16.0.0 0.0.31.255 area 0

default-information originate

redistribute static subnets

router bgp 100

no synchronization

bgp dampening

network 172.16.0.0 mask 255.255.192.0

neighbor 172.16.1.2 remote-as 100

neighbor 172.16.1.2 version 4

neighbor 192.68.1.2 remote-as 200

neighbor 192.68.1.2  version 4

neighbor 192.68.1.2 distribute-list 180 in

neighbor 192.68.1.2 distribute-list 180 out

neighbor 192.68.1.2 route-map NSP1-IN in

neighbor 192.68.1.2 route-map NSP1-OUT out

neighbor 192.68.1.6 remote-as 300

neighbor 192.68.1.6  version 4

neighbor 192.68.1.6 distribute-list 180 in

neighbor 192.68.1.6 distribute-list 180 out

neighbor 192.68.1.6 route-map NSP2-IN in

neighbor 192.68.1.6 route-map NSP2-OUT out

no auto-summary

ip as-path access-list 3 deny _100$

ip as-path access-list 3 permit *.

ip as-path access-list 4 deny _7597_

ip as-path access-list 4 permit _100$

ip route 172.16.0.0 255.255.192.0 Null0

route-map NSP1-IN permit 10

match as-path 3

route-map NSP1-OUT permit 10

match as-path 4

route-map NSP2-IN permit 10

match as-path 3

route-map NSP2-OUT permit 10

match as-path 4

Config Example 3 - BGP-OSPF-BGP

Figure 5 - ISP with EBGP to IXP

Figure 6  - iIX Detailed Diagram

ROUTER CONFIGURATIONS

Router iIX-TEL  (Cisco 7000 RP/SP with x RAM)

Domain Name	iix-tel.iix.id

!

no service finger

service nagle

no service slave-log

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

service compress-config

no service udp-small-servers

no service tcp-small-servers

!

hostname iIX-TEL

!

clock timezone Singapore 8

boot system flash xxxxxxxxxxxx				! Need to add IOS Image

aaa new-model

aaa authentication login APJII local

enable secret 5 <removed>					! Need to include password

!

username APJII1 password 7 xxxxxxxxxxxxxxxxxx		! APJII IX admin passwords

username APJII2 password 7 xxxxxxxxxxxxxxxxxx

username APJII3 password 7 xxxxxxxxxxxxxxxxxx

username APJII4 password 7 xxxxxxxxxxxxxxxxxx

username APJII4 password 7 xxxxxxxxxxxxxxxxxx

username APJII5 password 7 xxxxxxxxxxxxxxxxxx

username APJII7 password 7 xxxxxxxxxxxxxxxxxx

ip subnet-zero

no ip source-route

ip spd enable

!

interface Loopback0

ip address 198.32.204.1 255.255.255.255

no ip redirects

no ip directed-broadcast

no ip proxy-arp

no ip mroute-cache

no ip route-cache

!

interface Serial 0/0

description iIX Link to SatelIndo Site

ip address 198.32.204.17 255.255.255.252

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 0/1

description iIX Link to IndoSat Site

ip address 198.32.204.21 255.255.255.252

no ip address

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 0/2

description cbn.net.id direct link

ip address 198.32.204.33 255.255.255.252

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 0/3

description mega.net.id direct link

ip address 198.32.204.37 255.255.255.252

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 0/4

description

no ip address

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 0/5

no ip address

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 0/6

no ip address

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 0/7

no ip address

no ip redirects

no ip directed-broadcast

no ip proxy-arp

shutdown

!

interface Ethernet 1/0

description Connection to Shared Medium in Telkom facilities

ip address 198.32.204.65 255.255.255.192

no ip redirects

no ip directed-broadcast

no ip proxy-arp

ip route-cache same-interface

!

router bgp 7597

no synchronization

no bgp client-to-client reflection

bgp dampening

redistribute connected

neighbor internal peer-group

neighbor internal remote-as 7597

neighbor internal version 4

neighbor internal distribute-list 180 in

neighbor internal distribute-list 180 out

neighbor external peer-group

neighbor external version 4

neighbor external distribute-list 180 in

neighbor external distribute-list 180 out

neighbor 198.32.204.2 peer-group internal

neighbor 198.32.204.3 peer-group internal

neighbor 198.32.204.38 remote-as 7597			! IBGP to MEGA.NET.ID Router

neighbor 198.32.204.38 version 4

neighbor 198.32.204.38 distribute-list 180 in

neighbor 198.32.204.38 distribute-list 180 out

neighbor 198.32.204.38 route-reflector-client		! IBGP Route Reflector

neighbor 198.32.204.38 route-map MEGA.NET.ID-OUT out

neighbor 198.32.204.34 remote-as 4787			! EBGP to CBN.NET.ID Router

neighbor 198.32.204.34 peer-group external

neighbor 198.32.204.34 route-map CBN.NET.ID-IN in

neighbor 198.32.204.66 remote-as 4622			! EBGP to INDO.NET.ID Router

neighbor 198.32.204.66 peer-group external

neighbor 198.32.204.66 route-map INDO.NET.ID-IN in

neighbor 198.32.204.67 remote-as 4855			! EBGP to PACIFIC.NET.ID Router

neighbor 198.32.204.67 peer-group external

neighbor 198.32.204.67 route-map PACIFIC.NET.ID-IN in

bgp cluster-id 1000

no auto-summary

!

!

ip domain-name iix-tel.iix.id

ip name-server 202.159.32.2  !ns1.indo.net.id

ip name-server 202.154.1.2   !ns1.rad.net.id

ip name-server 202.147.252.2 !ns1.bit.net.id

ip classless

!

ip as-path access-list 51 permit _4622$

ip as-path access-list 51 permit _4855$

ip as-path access-list 51 permit _4761$

ip as-path access-list 51 permit _4795$

ip as-path access-list 51 permit _7597$

!

ip as-path access-list 52 permit _4787$

ip as-path access-list 52 permit _4855$

ip as-path access-list 52 permit _4761$

ip as-path access-list 52 permit _4795$

ip as-path access-list 52 permit _7597$

!

ip as-path access-list 53 permit _4787$

ip as-path access-list 53 permit _4622$

ip as-path access-list 53 permit _4761$

ip as-path access-list 53 permit _4795$

ip as-path access-list 53 permit _7597$

!

ip as-path access-list 55 permit _4787$

ip as-path access-list 55 permit _4622$

ip as-path access-list 55 permit _4761$

ip as-path access-list 55 permit _4795$

ip as-path access-list 55 permit _4855$

ip as-path access-list 55 permit _7597$

!

ip as-path access-list 71 permit _4787$

!

ip as-path access-list 72 permit _4622$

!

ip as-path access-list 73 permit _4855$

!

logging buffered 16384

logging trap debugging

logging source-interface Loopback0

logging 198.32.204.126						! Verify a logging server

access-list 1 permit XXXXXXXXXXXX				! Need a SNMP Server

access-list 1 deny all log					! Log all violations

access-list 5 permit 198.32.204.2

access-list 5 permit 198.32.204.3

access-list 5 permit 198.32.204.34

access-list 5 permit 198.32.204.38

access-list 5 deny all log					! Log all violations

!

access-list 100 permit ip host 202.158.2.130 any log

access-list 100 permit ip host 202.158.2.1 any log

access-list 100 permit ip host 198.32.204.34 any log

access-list 100 permit ip host 202.159.60.6 any log

access-list 100 permit ip 171.68.0.0 0.0.255.255 any	 log ! Allow me to login from Cisco

access-list 100 deny   ip any any log				! Log all violations

!

access-list 180 deny   ip host 0.0.0.0 any

access-list 180 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 180 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 180 permit ip any any

!

route-map CBN.NET.ID-OUT permit 10

match as-path 51

!

route-map INDO.NET.ID-OUT permit 10

match as-path 52

!

route-map PACIFIC.NET.ID-OUT permit 10

match as-path 53

!

route-map MEGA.NET.ID-OUT permit 10

match as-path 55

!

route-map CBN.NET.ID-IN permit 10

match as-path 71

!

route-map INDO.NET.ID-IN permit 10

match as-path 72

!

route-map PACIFIC.NET.ID-IN permit 10

match as-path 73

!

snmp-server community apjii-IIX RO 1		! Need to a harder to guess string

snmp-server trap-authentication

snmp-server enable traps config

snmp-server enable traps envmon

snmp-server enable traps bgp

snmp-server host XXXXXXXXXXXX apjii-IIX		! Need a SNMP Server

!

banner motd ^CC

APJII - iIX - iix-tel.iix.id

Unauthorized Access Prohibited ^C

!

line con 0

line aux 0

modem InOut

flowcontrol hardware

line vty 0 4

access-class 100 in

login authentication APJII

!

ntp clock-period 17179834

ntp authentication-key iIX md5 APJII

ntp authenticate

ntp trusted-key iIX

ntp source Loopback0

ntp access-group peer 5

ntp peer 198.32.204.2

ntp peer 198.32.204.3

ntp peer 198.32.204.34

ntp peer 198.32.204.38

ntp peer 198.32.204.66

ntp peer 198.32.204.67

end

Router iIX-IND  (Cisco 2501  with x RAM)

Domain Name	iix-ind.iix.id

!

no service finger

service nagle

no service slave-log

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

service compress-config

no service udp-small-servers

no service tcp-small-servers

!

hostname iIX-IND

!

clock timezone Singapore 8

aaa new-model

aaa authentication login APJII local

enable secret 5 <removed>					! Need to include password

!

username APJII1 password 7 xxxxxxxxxxxxxxxxxx		! APJII IX admin passwords

username APJII2 password 7 xxxxxxxxxxxxxxxxxx

username APJII3 password 7 xxxxxxxxxxxxxxxxxx

username APJII4 password 7 xxxxxxxxxxxxxxxxxx

username APJII4 password 7 xxxxxxxxxxxxxxxxxx

username APJII5 password 7 xxxxxxxxxxxxxxxxxx

username APJII7 password 7 xxxxxxxxxxxxxxxxxx

ip subnet-zero

no ip source-route

ip spd enable

!

interface Loopback0

ip address 198.32.204.3 255.255.255.255

no ip redirects

no ip directed-broadcast

no ip proxy-arp

no ip mroute-cache

no ip route-cache

transmit-buffers backing-store

!

interface Serial 0

description iIX Link to SatelIndo Site

ip address 198.32.204.25 255.255.255.252

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 1

description iIX Link to Telkom Site

ip address 198.32.204.22 255.255.255.252

no ip address

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

!

interface Ethernet 0

description Connection to Shared Medium in Indosat facilities

ip address 198.32.204.129 255.255.255.192

no ip redirects

no ip directed-broadcast

no ip proxy-arp

ip route-cache same-interface

!

router bgp 7597

no bgp synchronization

no bgp client-to-client reflection

bgp dampening

redistribute connected

neighbor internal peer-group

neighbor internal remote-as 7597

neighbor internal version 4

neighbor internal distribute-list 180 in

neighbor internal distribute-list 180 out

neighbor external peer-group

neighbor external version 4

neighbor external distribute-list 180 in

neighbor external distribute-list 180 out

neighbor 198.32.204.1 peer-group internal

neighbor 198.32.204.2 peer-group internal

neighbor 198.32.204.130 remote-as 4761			! EBGP to INDOSAT.NET.ID Customers

neighbor 198.32.204.130 peer-group external

neighbor 198.32.204.130 route-map INDOSAT1.NET.ID-IN in

neighbor 198.32.204.131 remote-as 4795			! EBGP to INDOSAT.NET.ID Router

neighbor 198.32.204.131 peer-group external

neighbor 198.32.204.131 route-map INDOSAT2.NET.ID-IN in

neighbor 198.32.204.132 remote-as 4434			! EBGP to INDOSAT.NET.ID Router

neighbor 198.32.204.132 peer-group external

neighbor 198.32.204.132 route-map RAD.NET.ID-IN in

bgp cluster-id 1000

no auto-summary

!

!

ip domain-name iix-tel.iix.id

ip name-server 202.159.32.2 					 !ns1.indo.net.id

ip name-server 202.154.1.2   					 !ns1.rad.net.id

ip name-server 202.147.252.2 					 !ns1.bit.net.id

ip classless

!

ip as-path access-list 54 permit _(4622|4855|4787|7597|4434)$

!

ip as-path access-list 55 permit _(4622|4855|4787|7597)$

!

ip as-path access-list 57 permit _(4622|4855|4787|7597|4434)$

!

ip as-path access-list 74 permit _4761$

!

ip as-path access-list 75 permit _4434$

!

ip as-path access-list 77 permit _4795$

!

logging buffered 16384

logging trap debugging

logging source-interface Loopback0

logging 198.32.204.62						! Verify a logging server

access-list 1 permit XXXXXXXXXXXX				! Need a SNMP Server

access-list 1 deny all log					! Log all violations

access-list 5 permit 198.32.204.2

access-list 5 permit 198.32.204.3

access-list 5 permit 198.32.204.34

access-list 5 permit 198.32.204.38

access-list 5 deny all log					! Log all violations

access-list 25 permit XXXXXXXXXXXXX			! Need to know login terminals

access-list 25 permit XXXXXXXXXXXXX

access-list 25 permit XXXXXXXXXXXXX

access-list 25 permit 171.68.0.0 0.0.255.255		! Allow me to login from Cisco

access-list 25 deny   any log				! Log all violations

!

access-list 180 deny   ip host 0.0.0.0 any

access-list 180 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 180 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 180 permit ip any any

!

route-map INDOSAT1.NET.ID-OUT permit 10

match as-path 54

!

route-map INDOSAT2.NET.ID-OUT permit 10

match as-path 57

!

route-map INDOSAT1.NET.ID-IN permit 10

match as-path 74

!

route-map RAD.NET.ID-IN permit 10

match as-path 75

!

route-map INDOSAT2.NET.ID-IN permit 10

match as-path 77

!

snmp-server community apjii-IIX RO 1		! Need to a harder to guess string

snmp-server trap-authentication

snmp-server enable traps config

snmp-server enable traps envmon

snmp-server enable traps bgp

snmp-server host XXXXXXXXXXXX apjii-IIX		! Need a SNMP Server

!

banner motd ^CC

APJII - iIX - iix-ind.iix.id

Unauthorized Access Prohibited ^C

!

line con 0

line aux 0

modem InOut

flowcontrol hardware

line vty 0 4

access-class 25 in

login authentication APJII

!

ntp clock-period 17179834

ntp authentication-key iIX md5 APJII

ntp authenticate

ntp trusted-key iIX

ntp source Loopback0

ntp access-group peer 5

ntp peer 198.32.204.2				! Peer with Satelindo Router

ntp peer 198.32.204.3				! Peer with Indosat Router

ntp peer 198.32.204.130			! Peer with indosat.net.id customer router

ntp peer 198.32.204.131			! Peer with indosat.net.id core router

end

Router iIX-SAT  (Cisco 2501  with x RAM)

Domain Name	iix-sat.iix.id

!

no service finger

service nagle

no service slave-log

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

service compress-config

no service udp-small-servers

no service tcp-small-servers

!

hostname iIX-SAT

!

clock timezone Singapore 8

aaa new-model

aaa authentication login APJII local

enable secret 5 <removed>					! Need to include password

!

username APJII1 password 7 xxxxxxxxxxxxxxxxxx		! APJII IX admin passwords

username APJII2 password 7 xxxxxxxxxxxxxxxxxx

username APJII3 password 7 xxxxxxxxxxxxxxxxxx

username APJII4 password 7 xxxxxxxxxxxxxxxxxx

username APJII4 password 7 xxxxxxxxxxxxxxxxxx

username APJII5 password 7 xxxxxxxxxxxxxxxxxx

username APJII7 password 7 xxxxxxxxxxxxxxxxxx

ip subnet-zero

no ip source-route

ip spd enable

!

interface Loopback0

ip address 198.32.204.2 255.255.255.255

no ip redirects

no ip directed-broadcast

no ip proxy-arp

no ip mroute-cache

no ip route-cache

transmit-buffers backing-store

!

interface Serial 0

description iIX Link to IndoSat Site

ip address 198.32.204.26 255.255.255.252

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

interface Serial 1

description iIX Link to Telkom Site

ip address 198.32.204.18 255.255.255.252

no ip address

no ip redirects

no ip directed-broadcast

no ip proxy-arp

!

!

interface Ethernet 0

description Connection to Shared Medium in Satelindo facilities

ip address 198.32.204.193 255.255.255.192

no ip redirects

no ip directed-broadcast

no ip proxy-arp

ip route-cache same-interface

!

router bgp 7597

no bgp synchronization

no bgp client-to-client reflection

bgp dampening

redistribute connected

neighbor internal peer-group

neighbor internal remote-as 7597

neighbor internal version 4

neighbor internal distribute-list 180 in

neighbor internal distribute-list 180 out

neighbor external peer-group

neighbor external version 4

neighbor external distribute-list 180 in

neighbor external distribute-list 180 out

neighbor 198.32.204.1 peer-group internal

neighbor 198.32.204.3 peer-group internal

neighbor 198.32.204.194 remote-as 7597			! IBGP to Satelindo Router

neighbor 198.32.204.194 peer-group external

neighbor 198.32.204.194 route-reflector-client		! IBGP Route Reflector

neighbor 198.32.204.194 route-map SATELINDO.NET.ID-OUT out

bgp cluster-id 1000

no auto-summary

!

!

ip domain-name iix-tel.iix.id

ip name-server 202.159.32.2 					 !ns1.indo.net.id

ip name-server 202.154.1.2   					 !ns1.rad.net.id

ip name-server 202.147.252.2 					 !ns1.bit.net.id

ip classless

!

ip as-path access-list 56 permit _4787$

ip as-path access-list 56 permit _4622$

ip as-path access-list 56 permit _4761$

ip as-path access-list 56 permit _4795$

ip as-path access-list 56 permit _4855$

ip as-path access-list 56 permit _7597$

!

logging buffered 16384

logging trap debugging

logging source-interface Loopback0

logging 198.32.204.62						! Verify a logging server

access-list 1 permit XXXXXXXXXXXX				! Need a SNMP Server

access-list 1 deny all log					! Log all violations

access-list 5 permit 198.32.204.2

access-list 5 permit 198.32.204.3

access-list 5 permit 198.32.204.34

access-list 5 permit 198.32.204.38

access-list 5 deny all log					! Log all violations

access-list 25 permit XXXXXXXXXXXXX			! Need to know login terminals

access-list 25 permit XXXXXXXXXXXXX

access-list 25 permit XXXXXXXXXXXXX

access-list 25 permit 171.68.0.0 0.0.255.255		! Allow me to login from Cisco

access-list 25 deny   any log				! Log all violations

!

access-list 180 deny   ip host 0.0.0.0 any

access-list 180 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 180 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 180 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 180 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 180 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 180 permit ip any any

!

route-map SATELINDO.NET.ID-OUT permit 10

match as-path 56

!

snmp-server community apjii-IIX RO 1		! Need to a harder to guess string

snmp-server trap-authentication

snmp-server enable traps config

snmp-server enable traps envmon

snmp-server enable traps bgp

snmp-server host XXXXXXXXXXXX apjii-IIX		! Need a SNMP Server

!

banner motd ^CC

APJII - iIX - iix-sat.iix.id

Unauthorized Access Prohibited ^C

!

line con 0

line aux 0

modem InOut

flowcontrol hardware

line vty 0 4

access-class 25 in

login authentication APJII

!

ntp clock-period 17179834

ntp authentication-key iIX md5 APJII

ntp authenticate

ntp trusted-key iIX

ntp source Loopback0

ntp access-group peer 5

ntp peer 198.32.204.2					! Peer with Satelindo Router

ntp peer 198.32.204.3					! Peer with Indosat Router

ntp peer 198.32.204.194				! Peer with Satelindo router

end

Router CBNnet config sample

AS Number	: 4787

IP Block	: 202.158.0.0/20

Peer to 	: IIX-Tel Serial 0/2 192.32.204.33/30

Interface IP	: CBN     Serial 5/7 192.32.204.34/30

!

no service finger

service nagle

no service slave-log

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

service compress-config

no service udp-small-servers

no service tcp-small-servers

!

hostname cbn

!

ip subnet-zero

no ip source-route

ip spd enable

!

interface Serial5/7

description dedicated link to IIX-TEL

ip address 198.32.204.34 255.255.255.252

ip access-group 195 in	! IP filtering from Internet

ip access-group 190 out	! IP filtering to Internet

no ip redirects

no ip directed-broadcast

no ip proxy-arp

no ip route-cache optimum

ip route-cache flow		! Turn on netflow ( only on 7xxx with RSP )

bandwidth 64

!

router bgp 4787

no synchronization

bgp dampening

network 202.158.0.0 mask 255.255.240.0

neighbor 198.32.204.33 remote-as 7597

neighbor 198.32.204.33 version 4

neighbor 198.32.204.33 route-map IIX-IMPORT in

neighbor 198.32.204.33 route-map IIX-EXPORT out

!

ip as-path access-list 3 permit ^7597_	! Permit any adv. origin from IIX

! Route advertisement filtering

! Filtering all the martian-ip and permit only own block ip

access-list 100 deny   ip host 0.0.0.0 any

access-list 100 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 100 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 100 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 100 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 100 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 100 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 100 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 100 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 100 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 100 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 100 deny   ip any 255.255.255.128 0.0.0.127

access-list 100 permit ip host 202.158.0.0 host 255.255.240.0

access-list 100 deny   ip any any log

!

! IP filtering to Internet

! Filtering all the martian-ip and permit only source from your own block ip

! This will prevent your customer from spoofing

!

access-list 190 permit tcp any any established

access-list 190 deny   ip 0.0.0.0 0.255.255.255 any

access-list 190 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 190 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 190 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 190 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 190 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 190 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 190 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 190 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 190 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 190 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 190 permit ip 202.158.0.0 0.0.15.255 any

access-list 190 deny   ip any any log

!

! IP filtering from Internet

! Filtering all the martian-ip and permit only destination to your own block ip

!

access-list 195 deny   ip 0.0.0.0 0.255.255.255 any

access-list 195 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 195 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 195 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 195 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 195 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 195 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 195 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 195 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 195 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 195 deny   ip 202.158.0.0 0.0.15.255 any log

access-list 195 permit ip any 202.158.0.0 0.0.15.255

access-list 195 deny   ip any any log

!

route-map IIX-IMPORT permit 10

match as-path 3

!

route-map IIX-EXPORT permit 10

match ip address 100

!

end

Router Indonet config sample

AS Number	: AS 4622

IP Block	: 202.159.0.0/18

Peer to 	: IIX-Tel Ethernet 2/0 192.32.204.65/26

Interface IP	: indonet Ethernet 0/1 192.32.204.66/26

!

no service finger

service nagle

no service slave-log

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

service compress-config

no service udp-small-servers

no service tcp-small-servers

!

hostname Indonet

!

ip subnet-zero

no ip source-route

ip spd enable

!

interface Ethernet0/1

description dedicated link to IIX-TEL

ip address 198.32.204.66 255.255.255.192

ip access-group 195 in	! IP filtering from Internet

ip access-group 190 out	! IP filtering to Internet

no ip redirects

no ip directed-broadcast

no ip proxy-arp

bandwidth 64

!

router bgp 4622

no synchronization

bgp dampening

network 202.159.0.0 mask 255.255.192.0

neighbor 198.32.204.66 remote-as 7597

neighbor 198.32.204.66 version 4

neighbor 198.32.204.66 route-map IIX-IMPORT in

neighbor 198.32.204.66 route-map IIX-EXPORT out

!

ip as-path access-list 3 permit ^7597_	! Permit any adv. origin from IIX

! Route advertisement filtering

! Filtering all the martian-ip and permit only own block ip

access-list 100 deny   ip host 0.0.0.0 any

access-list 100 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 100 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 100 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 100 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 100 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 100 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 100 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 100 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 100 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 100 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 100 deny   ip any 255.255.255.128 0.0.0.127

access-list 100 permit ip host 202.159.0.0 host 255.255.192.0

access-list 100 deny   ip any any log

!

! IP filtering to Internet

! Filtering all the martian-ip and permit only source from your own block ip

! This will prevent your customer from spoofing

!

access-list 190 permit tcp any any established

access-list 190 deny   ip 0.0.0.0 0.255.255.255 any

access-list 190 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 190 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 190 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 190 deny   ip 192.168.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 190 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 190 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 190 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 190 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 190 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 190 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 190 permit ip 202.159.0.0 0.0.63.255 any

access-list 190 deny   ip any any log

!

! IP filtering from Internet

! Filtering all the martian-ip and permit only destination to your own block ip

!

access-list 195 deny   ip 0.0.0.0 0.255.255.255 any

access-list 195 deny   ip 127.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 195 deny   ip 10.0.0.0 0.255.255.255 255.0.0.0 0.255.255.255

access-list 195 deny   ip 172.16.0.0 0.15.255.255 255.240.0.0 0.15.255.255

access-list 195 deny   ip 192.0.2.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 195 deny   ip 128.0.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 195 deny   ip 191.255.0.0 0.0.255.255 255.255.0.0 0.0.255.255

access-list 195 deny   ip 192.0.0.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 195 deny   ip 223.255.255.0 0.0.0.255 255.255.255.0 0.0.0.255

access-list 195 deny   ip 224.0.0.0 31.255.255.255 224.0.0.0 31.255.255.255

access-list 195 deny   ip 202.159.0.0 0.0.63.255 any log

access-list 195 permit ip any 202.159.0.0 0.0.63.255

access-list 195 deny   ip any any log

!

route-map IIX-IMPORT permit 10

match as-path 3

!

route-map IIX-EXPORT permit 10

match ip address 100

!

end

FURTHER READING AND REFERENCES

Here is some suggested reading for further reading:

Internet Routing Architectures, New Riders Publishing (Cisco Press). ISBN 1-56205-652-2. Author: Bassam Halabi.

Using the Border Gateway Protocol for Interdomain Routing, Cisco Connection On-line (CCO) Web site: http://www.cisco.com/univercd/data/doc/cintrnet/ics/icsbgp4.htm

Internetworking Technology Overview. On-line whitepapers and  tutorials on the essentials of routing and switching. Cisco Connection On-Line (CCO) Web and the Cisco Documentation CD: http://www.cisco.com/univercd/data/doc/cintrnet/75818.htm

Technology Information and Whitepapers.  Key references and practical internetworking examples. Cisco Connection On-Line (CCO) Web and the Cisco Documentation CD: http://www.cisco.com/univercd/data/doc/cisintwk.htm

Hopefully no one is still using RIPv1. RIP version 1 is a historical protocol that was replaced by RIP version 2. Since the Internet requires CIDR. All ISPs should be using a CIDR compliant routing protocol.

Route flapping got so bad that a new BGP technique has to be developed to protect - route flap damping. Cisco's IOS has a BGP feature - bgp dampening. See the Cisco documents for more details.