*******************************************************************

Computer can now play role of CW keyer. All you need is paddle and

simple CW interface connected to your serial port.

...................................................................

This CW TSR utility has been primarily written for my friend s53ww - Robi

who wrote the VHFCTEST program, which is very popular in our region. You can

download the latest version of the VHFCW and VHFCTEST from the

ftp server:

ftp.hamradio.si

There are two basic programs for CW-TSR :

CW TSR utility VHFCW.EXE

CW TSR monitor : CWMONI.EXE

The VHFCW remains active in your computer.

Syntax:

VHFCW X {M} {P} {S} {I}

X=COM (1,2) must be defined

M=Monitor (0,1)

P=Paddle interface connected (0,1)

S=Squeeze (0,1);

I=Soft. Interrupt no. ($60-$FF), default $60

The CWMONI is a simple monitoring program which enables you to

test the TSR utility and change different settings.

Syntax:

CWMONI {I}

I=software interrupt nr., $60 by default

If CONFIG.SYS includes many device drivers then there is possibility that

computer hangs (immediately after running VHFCW or even during contest work).

The only solution is to clean up CONFIG.SYS and maybe also AUTOEXEC.BAT.

Interface:

____________

2/DB25 (3/DB9)        I\  I

o----------I  \I-------o ------I

I / I       I       I

I   +   I

=======    I

\_________/  I   tantal 10 uf/16 V

I       I

7/DB25 (5/DB9)                    I   -   I

o----------------------I       I

paddle - center                       I

I

1 k 5                 I

6/DB25 (6/DB9)     ______                 I

o------I______I----------------o

paddle - dit                          I

I

I

1 k 5                 I

8/DB25 (1/DB9)     ______                 I

o------I______I----------------I

paddle - dash

1k               opto coupler 4N..

20/DB25 (4/DB9) ______

o---I______I------

I

I          ___________o

__I__   II  /

\   /   II/               CW TX key

__Y__   II\

7/DB25 (5/DB9)              I     II  \>----------o

o_________________I

1k               opto coupler 4N..

4/DB25 (7/DB9)  ______

o---I______I------

I

I          ___________o

__I__   II  /

\   /   II/                PTT line

__Y__   II\

7/DB25 (5/DB9)              I     II  \>----------o

o_________________I

You can also use transistors instead of opto-isolators,

but this is better approach...

Paddle should be connected in parallel to connector contacts (7 for center

contact, 6 for dots and 8 for dashes). Complete circuit can be easily built

into plastic body of DB25 connector and with little effort also into DB9.

(-----------------------------------------------------------)

Information for programmers

(-----------------------------------------------------------)

Communication with VHFCW

To communicate with vhfcw tsr the software interrupt $60 is used by

default. You can change it with different options in command

line when running VHFCW.

!!! The AH register should always be $30 for identification. !!!

function        AL reg      BX reg        CX reg      comment

CW SPEED          1           X (Ch/min, 15..260)

PTT ON-OFF        2

QRQ               3

QRS               4

Send text         5           Seg(string) Offs(string)

PTT DELAY         6           X [mS]

DITX              7           X [ dash/dot  *10 ]     default  30

PCHX              8           X [ dot /pause*10 ]     default  10

PICX              9           X [ dot /ch.pause  *10] default  30

VOXDELAY         10           X [mS]

MEMKEYER         11           X [1,0]                 squeeze on/off

CWMONITOR        12           X [1,0]                 cwmonitor on/off

PADDLE           13           X [1,0]                 paddle iface connected

ACTIVE MSG.      14                                   Writes 'ACTIVE' msg.

INTERR. ON       15                                   Activate TSR

INTERR. OFF      16                                   Deactivate TSR

TONE             17           X [50-2000]             Monitor Fq

additional programs

cwkey.exe - simple program to control TSR

Note:

Usually, when you run VHFCW you get a message

Soft. interrupt OK ...

If you get message software interrupt in use, that means that interrupt

$60 is already in use by other programs. In general this should

not cause any problems, but in case it does, you better change this

value to an unused SW interrupt. (commands sysinfo or similar utility..)

Description of commands

------------------------

CW SPEED   (Ch/min, 15..260)

AL=1, BX=x

x=speed in characters/min (approximation)

PTT ON-OFF

AL=2  Toggle PTT pin on/off

If the computer transmits any text or is in repeat mode (@X) command,

this mode will be interrupted.

QRQ

AL=3

Increase CW speed

QRS

AL=4

Decrease CW speed

Send text

AL=5

BX=Seg(string)

CX=Offs(string)

Transmit text, address of text is passed with BX and CX registers.

The variable should be of type string. Before text is transmitted,

the ptt goes high, waits pttdelay, then transmission is started.

The repeat command is also available, you can put @X command

at the end of the text, that means that the text should be repeated

every X seconds. You can stop transmitting text with PTT function

or if there is a paddle connected, then touching

the paddle will stop text transmission and keying will be proceeded

as in manual keying mode.

example: CQ CQ DE S57RA @5

(this text will be transmitted with 5 second breaks)

PTT DELAY

AL=6

BX=x [mS]

Definition of PTT DELAY - time to wait after PTT goes HIGH to start CW

signal (to save some money on preamplifier's FETs or GaAs-fets).

DITX

AL=7

BX=x

x = dash/dot ratio * 10, default  30. Some contesters prefer higher

dash/dot ratio than 3, usually 3.2 or 3.4.

PCHX

AL=8

BX=x

x= dot /pause ratio *10, default  10

Pause in character ratio.

PICX

AL=9

BX=x

x= dot /ch.pause ratio *10, default  30

Pause between two characters.

VOXDELAY

AL=10

BX=X [mS]

This parameter defines how many mS the PTT signal remains HIGH and waits

for the next character - in manual transmit mode only.

MEMKEYER

AL=11

BX=x

x=[1,0] squeeze (iambic keying) on/off

CWMONITOR

AL=12

BX=x

x=[1,0] PC speaker monitor on/off

PADDLE

AL=13

BX=x

x=[1,0] paddle interface connected. If this parameter is set and you

don't have the interface connected, then you get something like

endless -.-.-.-.-.-.-. etc...

ACTIVE MSG.

AL=14

The tsr writes 'ACTIVE' msg. and is for testing only.

INTERRUPTS ON

AL=15

Activate com port interrupts (if they have been stopped with

function 16)

INTERRUPTS OFF

AL=16

Deactivate TSR by stopping comm port interrupts. The program is

still resident in memory and can be reactivated with function 15.

TONE

AL=17

BX=x

x=[50-2000] Monitor Fq in Hertz, default 500.

Any questions or suggestions are welcome to my PR or EMAIL address.

73 & GL Pavle,

S57RA @ S50BOX.SVN.EU

pavel.reberc@guest.arnes.si