This is an explanation of how to use the Pawel  Jalocha SP9VRC   mfsk  files

mfsk_tx and mfsk_rx and mfsk_trx  and the

Olivia-Aid   files by Chris Watts VK3DNH

THE ABOVE FILES HAVE BEEN TESTED WITH W2000 AND WINXP

W98 users will not be able to use these files – Chris is working on doing

Another program for W98 users

These help files try to explain how to use the Olivia-Aid program to make the operation a little easier .

Run the program by clicking on the file

OliviaAid.exe   or make a shortcut to this file and put it on your desktop

When executed this will bring up

BUTTON EXPLANATIONS

The T box figures are the number of tones you want to use   2 4 8 16 (32) 64 128 256

The BW box is the bandwidth in hz   - 125 250 500 (1000) 2000

The M box is the setting for how wide the receiver will look for the signal

default   is 16

=======================================================

The M value may vary from 2 to 156...

Here some combinations of M and resulting tuning tolerance.

ALL IS VALID ONLY FOR T=32 and BW= 1000.

For other T and BW the tuning tolerance is different.

M   Tuning tolerance (Hz)

2            +/-   31.2

4                  62.5

8                 125.0

16                 250.0

32                 500.0

64                1000.0

128                2000.0....

The wider the tuning tolerance is the more calculation power is needed.

---------------------------------------------------------------------

Right click on “T” brings up the Buffer window

Left click on “BW” will bring up the version box

Right click on “BW” bring up a window displaying all of the .log files in the current directory

Right click on TX Brings up the Com port selection box

If you wish to use   Vox   operation – leave the box blank

Vox is the easiest to use -   when     all text or files in the Transmit window   are transmitted it will automatically return to receive

IF you do set a com port number – REMEMBER   that you must WAIT until all of

Transmit window contents are displayed in the Receive window (plus a little bit extra) before you click on the RX button

IF YOU CLICK THE RX BUTTON PRIOR TO THIS IT WILL IMMEDIATELY RETURN TO RECEIVE

Single screen operation

Separate TX and RX screens

Click on   RX   to bring up the receive screen   – close by clicking again on the RX button

For those that only want to receive

There is no provision to set any soundcard offset when using

The mfsk_tx and mfsk_rx files

Right click on TX and it will bring   up the com port selection box – enter

the   com port number –  if you wish to use VOX leave the box blank

Click on T X and   Transmit and Receive windows will come up and the program will begin transmitting ----   type to the transmit window   or load  from the buffer – when finished click on  the  RX button to go back to the Rx screen

These are the basic principles of  single screen operation

-------------------------------------------------------------------------------------------------------

Next for operation using the mfsk_trx.exe file

Right click on the TX box and it will bring up the com port selection box

enter the port number – then click ok   - or if you want to use VOX  make sure there is no entry in the com port setup box

If you do decide to use a com port   - REMEMBER - that if you click on the RX button

when  in transmit it will  drop the PTT   - you have to wait until you see

all   of the sent text appear in the RX window (plus a little extra time) before clicking on the RX button

Click on the TRX button – a sound card setting box will appear – default is 8000

You can edit this box to reflect any TX offset that you may have

Click OK and it will bring up   the Split Screen   terminal and the program will be in receive at this point

To transmit – click on the TX button and type to the Transmit or load a file -  when finished typing click on RX    button to return to the receive screen

If using VOX operation it will automatically   return to receive when all of the contents of the transmit window are sent   - If using PTT do not hit the RX button until all of the

Transmit window contents have been displayed in the Receive screen

That is the basic operation procedure for using the TRX file

Picture below shows my setup here with Gram running s well as the TRX

Screen and also OliviaAid   and the Buffer window open

Now for the Buffer operation

You can type into the Transmit window even while you are receiving to have text

ready   to transmit or you can use the Buffer window  - use of the buffer is recommended since  you can use Backspace there – but you cannot use backspace from the Transmit window

Right click on the “T” to bring up the Buffer

You can type directly to the Buffer window

or  select a file by clicking on   LOAD

Brings up the file selection selection  box – select the file you want to transmit – click OK - it will automatically load to the Buffer   window   (you can also copy and paste any text   from anywhere on your computer in text as well)  You also have use of the backspace function if you use the OliviaAid  buffer

YOU CANNOT USE THE BACKSPACE FUNCTION DIRECTLY FROM THE TRANSMIT WINDOW

You can send one lot of text/files from the buffer to the Transmit window

by clicking on the SEND READY FOR TX button

The text loaded to the Buffer window will disappear when you click the SEND READY FOR TX   button and    will   be  pasted  to the  Transmit  window   and you can then use the buffer again for more text or files and again send this to the Transmit and so on

--- then click TX to transmit   - you can also type directly   into the transmit window while transmitting if you wish  -  when finished typing or loading files click   RX which will take you back to receive when the  Transmit window contents are finished

Buttons F2 thru F6 can be used for storing pre-prepared items such as CQ – station details etc

To save messages to the F key locations you have to do the following

Place the Cursor into the Buffer text window

CTL plus F key number

Type into the window that appears the text you want in that F key location

Hit Escape key

Hit SHIFT plus the F key number

Brings up a box in which you can enter the name for the F key

Click OK  - it is then saved to the COMSettings file  with a   .dnh   extension

All you need to do then to send   the  contents  is click on the F2 (etc)  button to paste it to the Buffer window

You can also hand type to the Buffer or load   text files from anywhere on the computer

You can also continue to type to the Buffer as well – when finished click on the

SEND READY FOR TX   and the contents will be pasted to the Transmit window and the buffer contents will be cleared

You can continue to add information to the buffer by typing or adding files or the F keys

But you must hit the SEND READY FOR TX button each time you want to sent the Buffer contents to the Transmit screen

When finished click the RX button

Right click on the BW

RIGHT click on any file and it will bring up a drop down menu in which you can do a number of functions

LEFT click on any of the .log files brings up a box

If you check the box you can save the file as   text file a in the     savedlogs

Folder   in Olivia Aid by closing the window

Regards

Les VK2DSG     12 JAN 2005