Patrick LINDECKER (F6CTE) the 26th of january 2003

SPECIFICATIONS OF THE PSKAM AND PSKAM10 MODES (MULTIPSK version 2.0)

Preliminary: PSKAM and PSKAM10 are experimental modes which specifications may evolve.

Digital communications are, sometimes, given impossible, because of:

* QRM of others radio-stations,

* fading,

* a too weak ratio Signal-to-Noise ratio (the signal is drowned in the noise) ,

* ionospheric Doppler modulation (for low baud speed) or time overlap between different paths (for quick baud speed)…

The aim of author, for PSKAM, is to allow communications in following conditions:

* “chat” communication between ham people (no file transfer) in short waves with much less errors that with PSK31,

* possible communication on a AF spectrum from 200 to 2500 Hz (almost the transmission band of a SSB receiver), with less than 2 % of false received characters, this for a Signal-to-Noise ratio (before filter) inferior or equal to the one obtained for PSK31 (-10 dB: signal 10 times less powerful than noise).

* possible communication under SW bad conditions,

* speed of transmission about 30 words/minute, in equivalence with CW, this speed corresponds to a quick typing for an OM,

* transceiver stable in frequency (same stability as for PSK31),

* PC available for the OM: Pentium 166 or more with a sound card (SOUND BLASTER compatible).

Comparison with PSK31: it is reminded that the PSK31 mode is designed to assure communications with ratio Signal-to-Noise until 0.1 (-9 dB), so a signal about 10 less powerful that noise. The equivalent Morse speed is 37 words/minute in capital and 51 words/minute in small letters.

The difference between the PSK31 (-9 dB) and PSKAM (-11 dB) is about 2 dB in Signal-to-Noise ratio.

For PSKAM10, the aim is doing a communication with a S/N down to –19 dB (signal 100 times less powerful than noise), accepting a slow typing speed (6 words/minute).

CHOICE OF THE COMMUNICATION MODE

First it must be determined what communication mode to choose, the most interesting with regards with the preceding hypothesis.

The most simple is a two phases modulation in a biorthonormal form: 0° phase and 180° phase (inverse). It must be seen as BPSK (binary PSK) or binary PAM. It is also a very good (but not the best) mode to transmit in white noise with a low probability of symbol error.

To avoid any phase reference, it will be chosen a differential mode (what is detected is not the phase but its inversion or not: 0°--->180 ° or 180°--->0° for inversion and 0°--->0 ° or 180°--->180° for non-inversion). Unfortunately, the bit error rate is doubled.

The detection remains coherent (in fact “ quasi-coherent ” because the synchronization is not sure but built with the signal).

Note: it’s called “coherent” because a PLL is used to follow the phase. But the phase is known with an ambiguity of 180 °, so it’s not a real “coherent” detection.

###### APPLICATION TO PSKAM

A first mode was proposed which name was PSK10: the transmission was done in 10 baud’s, D-2PSK (Differential two-phases shift keying). A Varicode dictionary was associated to this mode. 10 baud’s (0.1 sec by bit) because this corresponds to 20 words/min in CW which is the minimum speed for a normal talk and because a frequency of 10 Hz would be easier to realize with a frequency reference based on a “second clock” (as transmitted by radio clocks, for example). This solution has been abandoned because, without correction, the duration of the bit (0,1 second) is too long in front of the ionospheric Doppler modulation in SW.

The time overlap between different paths makes problem for 100 bauds mode and above. So, for the SW, a reasonable speed is about 30/60 bauds. A first chosen speed was 31,25 bauds (MULTIPSK 1.7 and 1.8) which is the BPSK PSK31 speed but is was a little slow for typing and speed has been increased to 50 bauds which gives a confortable speed of 31 words/minute.

To simplify, the type of filter used is the one used for PSK 31 (raised cosine). The envelope of the level is constant (except during transitions) so the average power is about 70 % of the maximum power (obtained for example with a “tune”).

As PSK31, the transmission may be done in LSB or USB. But to simplify, USB is recommended so when a frequency is proposed everybody speak of the same one.

###### APPLICATION TO PSKAM10

The duration of the bit (0,1 second) is too long in front of the ionospheric Doppler modulation in SW. Although, as there is a repetition of the character, the probability of error due to this phenomenon is very reduced so errors are mainly due to noise.

The type of filter used for sending bits has been chosen after some experimentations.

I finally choose a special shape of pulse: rectangular but the connection between 2 successive different bits is done by a decreasing sine then by a creasing sine (each of one during a quarter of period). This allows a not too large bandwidth without interference between symbols. Except during transitions, the envelope of the level is constant so the average power is 85 % of the maximum power (obtained for example with a “tune”).

Example: 1011:   1    |   0   |  1    |   1

R-R-R-D-C-R-R-D-C-R-R-R-R-R-R-R

or C                          or D

R: rectangular (=1) of ¼ T. D: Sine decreasing of ¼ T (1 to 0)

C: Sine creasing of ¼ T (0 to 1)

The transmission may be done in LSB or USB. But to simplify, USB is recommended so when a frequency is proposed everybody speak of the same QRG.

BITS DISPOSITION AND DICTIONARY

For bits disposition, it’s inspired from AMTOR (fixed length). But to avoid the problem to shift between letters and figures (which drives to errors in packet), it has been chosen to create an original code of 8 bits.

Note: I have tried, first, a Varicode: it’s not the solution...a lot of errors due to a complex interpretation.

This one is 70 characters long in place of 35 in AMTOR.

Each character contains 5 “0” and 5 “1”, which eliminates 256-56=200 invalid characters.

The reception of bits is done from left to right (“ 0 ” is a phase change and “ 1 ” a continuous phase as in PSK31).

With 8 bits length and 31.25 bauds, the speed in 19,53 wpm (31.25/8/2*60/6) which is the minimum reasonable.

'A': DEFINITION:='01100100';

'B': DEFINITION:='00100101';

'C': DEFINITION:='11000001';

'D': DEFINITION:='00001110';

'E': DEFINITION:='00100110';

'F': DEFINITION:='11000010';

'G': DEFINITION:='00001101';

‘H': DEFINITION:='00100011';

'I': DEFINITION:='11000100';

'J': DEFINITION:='00001011';

'K': DEFINITION:='01100010';

'L': DEFINITION:='11001000';

'M': DEFINITION:='00010110';

'N': DEFINITION:='01010010';

'O': DEFINITION:='10110000';

'P': DEFINITION:='10000110';

'Q': DEFINITION:='01010100';

'R': DEFINITION:='11100000';

'S': DEFINITION:='00010011';

'T': DEFINITION:='01011000';

'U': DEFINITION:='10100001';

'V': DEFINITION:='00011001';

'W': DEFINITION:='01001001';

'X': DEFINITION:='10100010';

'Y': DEFINITION:='10010100';

'Z': DEFINITION:='01001010';

'-': DEFINITION:='00011100';

'?': DEFINITION:='01001100';

':': DEFINITION:='10100100';

'$': DEFINITION:='10001010';

'3': DEFINITION:='01000101';

'!': DEFINITION:='10101000';

'@': DEFINITION:='00110001';

' ': DEFINITION:='01000110';

'8': DEFINITION:='11010000';

'%': DEFINITION:='00110010';

'(': DEFINITION:='01000011';

')': DEFINITION:='10010001';

'.': DEFINITION:='00110100';

',': DEFINITION:='01100001';

'9': DEFINITION:='10010010';

'0': DEFINITION:='00111000';

'1': DEFINITION:='01010001';

'4': DEFINITION:='00011010';

''': DEFINITION:='00101001';

'5': DEFINITION:='10000101';

'7': DEFINITION:='00010101';

'=': DEFINITION:='00101010';

'2': DEFINITION:='01101000';

'/': DEFINITION:='10001001';

'6': DEFINITION:='00101100';

'+': DEFINITION:='01110000';

CHR(13):DEFINITION:='10001100';

CHR(8):DEFINITION:='10000011';{Error reset key}

'“': DEFINITION:='00000111';{phasing 1...displayed as ‘”‘}

'”': DEFINITION:='10011000';{phasing 2...not displayed}

‘“‘: phasing signal 1 and ‘”’: phasing signal 2

CHR(8): back space (Error reset key)

CHR(13): <CR> + <LF> (Carriage return + Line feed)

The small letters keystroke at the editor are translated in capital letters.

IMPORTANT: in the same order, all the accentuated letters are translated in the equivalent non-accentuated letter. For example,    é and É become E.

Precisely, here is a part of the program (PASCAL) for reception of characters:

CARAC_RECU is the received character, REUSSITE the flag to specify if the

character is from PSKAM character and CARAC_A_AFFICHER is the character to display.

PROCEDURE VERIF_CARACTERE_ET_TRADUCTION_PSKAM(CARAC_RECU:CHAR;

VAR REUSSITE:BOOLEAN;VAR CARAC_A_AFFICHER:CHAR);

BEGIN

REUSSITE:=FALSE;

CARAC_A_AFFICHER:=CHR(0);

{a...z}

IF (CARAC_RECU>='a') AND (CARAC_RECU<='z') THEN

BEGIN

REUSSITE:=TRUE;

CARAC_A_AFFICHER:=CHR(ORD(CARAC_RECU)-32);

EXIT;

END;

IF (CARAC_RECU='Ç') OR (CARAC_RECU='ç') THEN CARAC_RECU:='C';

IF (CARAC_RECU='ü') OR (CARAC_RECU='û') OR (CARAC_RECU='ù') OR

(CARAC_RECU='Ü') OR (CARAC_RECU='ú')    THEN CARAC_RECU:='U';

IF (CARAC_RECU='é') OR (CARAC_RECU='ê') OR

(CARAC_RECU='æ') OR (CARAC_RECU='Æ') OR (CARAC_RECU='ë') OR

(CARAC_RECU='è') OR (CARAC_RECU='É')    THEN CARAC_RECU:='E';

IF (CARAC_RECU='â') OR (CARAC_RECU='ä') OR (CARAC_RECU='à') OR

(CARAC_RECU='å') OR (CARAC_RECU='Ä') OR (CARAC_RECU='Å')

OR (CARAC_RECU='á') THEN CARAC_RECU:='A';

IF (CARAC_RECU='ï') OR (CARAC_RECU='î') OR (CARAC_RECU='ì') OR

(CARAC_RECU='í') THEN CARAC_RECU:='I';

IF (CARAC_RECU='ô') OR (CARAC_RECU='ö') OR (CARAC_RECU='ò') OR

(CARAC_RECU='Ö') OR (CARAC_RECU='ó')    THEN CARAC_RECU:='O';

IF (CARAC_RECU='ÿ') THEN CARAC_RECU:='Y';

IF (CARAC_RECU='ø') OR (CARAC_RECU='Ø') THEN CARAC_RECU:='0';

IF (CARAC_RECU='ƒ') THEN CARAC_RECU:='F';

IF (CARAC_RECU='¿') THEN CARAC_RECU:='?';

IF (CARAC_RECU='ñ') OR (CARAC_RECU='Ñ') THEN CARAC_RECU:='N';

IF ((CARAC_RECU>='A') AND (CARAC_RECU<='Z')) OR

((CARAC_RECU>='0') AND (CARAC_RECU<='9')) OR

(CARAC_RECU='-') OR (CARAC_RECU='?') OR    (CARAC_RECU=':') OR

(CARAC_RECU='$') OR    (CARAC_RECU='!') OR (CARAC_RECU='@') OR

(CARAC_RECU=' ') OR    (CARAC_RECU='%') OR (CARAC_RECU='(') OR

(CARAC_RECU=')') OR    (CARAC_RECU='.') OR (CARAC_RECU=',') OR

(CARAC_RECU='''') OR (CARAC_RECU='=') OR    (CARAC_RECU='/') OR

(CARAC_RECU='+') OR (CARAC_RECU=CHR(13)) OR (CARAC_RECU=CHR(8))

OR (CARAC_RECU='“')    THEN

BEGIN

REUSSITE:=TRUE;

CARAC_A_AFFICHER:=CARAC_RECU;

END;

END;

The idle character '“' is transparent for the user. The Error reset key corresponds to the "Back Space" (CHR(8)).

# DESCRIPTION OF THE PROTOCOL AND TREATMENT

Preliminary: The protocol is very similar to the one used in AMTOR FEC (Mode B) in Collective Mode.

##### THE STATION SENDING PSKAM

- Emits each character twice: the first transmission (DX) of a specific character is followed by the transmission of four other characters, after which the retransmission (RX) of the first character takes place, allowing for time diversity reception at 0.8 s time space for PSKAM and 4 s for PSKAM10.

- Emits as a preamble to messages , alternately, twice, the phasing signal 1 and the phasing signal 2 whereby phasing signal 1 is transmitted in the DX, and phasing signal 2 in the RX position. After the pair of phasing signal 1/ phasing signal 2, it is emitted a Carriage Return (CHR(13)) to pass at the next line.

- Emits during the breaks between two messages in the same transmission the phasing signals 1 and the phasing signals 2 in the DX and the RX position, respectively.

##### THE STATION RECEIVING PSKAM

##### Check both characters (DX and RX):

##### * If they are identical and valid: it is printed the character,

##### * If they are identical and invalid: it is printed ‘¿’ (error)…the more probable is a phasing lost,

##### * If the DX character is valid, the one in RX position is invalid, it is printed the DX character,

##### * If the DX character is invalid, the one in RX position is valid, it is printed the RX character,

##### * If the RX and DX characters are valid but different (and different from the couple phasing signal 1 / phasing signal 2), a search of the most probable character based on the levels of reception after the base band demodulation must be done: it is printed the most probable valid character,

##### * If the RX character is invalid, the one in RX position is invalid, a search of a possible valid character based on the levels of reception after the base band demodulation must be done: after which, depending on result, it is printed either a valid character or an error ‘¿’.

Note 1: the question of the treatment of resynchronization according to the number of errors is not fixed, but the goal is, of course, not to lose characters.

Note 2 : to take into account QSB and QRM which may do lose synchronization during several seconds, the software must continue to translate (with the preceding synchronization) during 10 seconds considering the loss of characters as transitional. After 10 seconds, if synchronization has not been recovered, it’s considered as lost and the translation stops.

##### PHASING

##### * On receipt of the sequence “ phasing signal 1 ” - “ phasing signal 2 ”    separated by 4 valid characters in which “  phasing signal 1 ” determines the DX and “  phasing signal 2 ” the RX position. As soon as the two phasing signals have been determined in good position, phasing is acquired. This type of phasing is used at the beginning of the transmission and during idle characters transmissions.

##### * If signal has been yet emitted and only normal characters are transmitted, the phasing

##### must be acquired by looking for a sequence of 6*8=48 bits filling the following conditions:

##### - each successive byte is in accordance to the format 5 bits to “ 0 ” and 5 bits to 	“ 1 ” ,

##### - the 8 last bits are identical to the 8 first bits.

##### END OF EMISSION

There is no specific “ end of emission signal ”. Normally, the ham who wants to finish transmits +K (invitation to transmit) or a VA or a SK (end of transmission).

But it is obvious that the last RX characters must be transmitted so, in the DX correspondent positions, a phasing signal 1 must be found.

For example, to transmit ABC, a pause (of one character duration) and then +K:

DX 		RX			DX is the first position

“		”			RX is the second position

“		”	 		“ is the phasing signal 1

CHR(13)	” 			” is the phasing signal 2

A		”

B		CHR(13)

C		A

“		B

+		C

K		”

“		+

“		K

DETECTION OF SIGNAL IN NOISE AND DISPLAY

One difficulty is to detect and display a very weak signal in noise. The only way to do it, is to average the spectrums. For example, in MULTIPSK, the displayed spectrum for PSK31 and PSKAM is an average of 12 spectrums (3 by second). As a signal (as PSKAM or PSK31) is coherent and noise is non-coherent, the average of noise becomes nil and the average of the signal remains constant, so the more the number of spectrums averaged is, the better the signal comes out from noise. The problem is in the introduction of a time delay (the averaging is a filter) so the phenomena’s becomes slow…the increasing and the decreasing of the signals are slow.

In conclusion, it’s a compromise between fine detection and time delay…and it’s one of the limit for detection of signal in noise: to pass to a limit of S/N ratio of 0.001 for example, it will be necessary to have a baud rate of 1 baud so a CW equivalent speed of 0.62 words/min.…and time delay of 30 seconds or more. It will be necessary for OM to be very patient!